Computer Networks – IT2050

Devindu Samarasinghe

Lecture 01 – Overview of configuration of Network Devices

Network Devices – Internal Components

CISCO IOS Software

- As with a computer, a network device cannot function without an operating system

- CISCO calls its OS as the CISCO international Operating System – CISCO IOS

Command Line Interface – CLI

- The CISCO IOS software uses a command line interface as the traditional console environment

- This environment is accessible through several connection methods

  - Console

  - Aux Port

  - Telnet – is a protocol that allows you to connect to remote computers (called hosts) over a TCP/ IP network (such as internet). Using telnet client software on your computer, you can make a connection to a telnet server (that is, remote host).

User Level Passwords

| Access from | Password Type | Configuration |
| --- | --- | --- |
| Console | Console password | line console() login password faith // password can be anything |
| Auxiliary | Auxiliary password | line aux() login password hope // password can be anything |
| Telnet | Vty password | Line vty 0 4 login password love // password can be anything |

Router Modes

Router Modes

Router Memory

Router Memory consist of

- RAM

- NVRAM

- Flash

- ROM

- Console

- Interfaces

RAM

- Store running or active configuration files

- Loses content when router is powered down

- A working storage

ROM

- Read only Memory

- Stores bootable IOS image and bootstrap program

NVRAM

- Provides storage for the startup configuration file

- Retains content when router is powered down

Flash Memory

- Holds the fully functional IOS Image

- Retains content when router is powered down

- Is a type of electronically erasable, programmable ROM (EEPROM)

Displaying Configuration files

show running-config  					show startup-config

Managing Configuration files

Copy Command

- Copy files in a router (configuration file, new version of the IOS software)

- Move configuration files among RAM, NVRAM and TFTP server

Copy running-config startup-config

copy running-config startup-config cont.